# Pitta Firmware Uploader

Uploader4Pitta is a firmware update tool for PITTA Base.

- Copy the Uploader folder to any directory on your PC

- Connect PC and Pitta base with USB serial cable

  - Run Windows Device Manager to check whether Arduino driver is installed

  - If the Arduino Driver is not installed and it is recognized as a different name in Other devices or Ports, you need to install the driver manually.

  - In the Device Manager, mouse right-click the COM port that was identified as the Arduino (the Arduino must be plugged into the computer first). Select Update Driver Software... from the menu that pops up.

  - Find the drivers folder. This folder is located in PittaUploader was saved.

  - In the Windows Security dialog box that pops up, click Install

  - Finally go back to Device Manager and check that the COM port is now identified as Arduino Micro.

- Open Windows Command Prompt at your PC

- Change directory to uploader folder

- Copy firmware.hex file to uploader folder

- Run "uploader4pitta.bat firmware.hex" at Command Prompt

- Upload progress displayed in the command prompt window

  - Detect Pitta's COM port detect

  - Reset Pitta MCU

  - Detect Bootloader's COM Port

  - Upload firmware.hex file (Read -> Writing -> Verifying) to Pitta Base

- Confirm upload complete